TX-3050SF Servo Flat Screen Printing Machine

★Scope of application

Suitable for high-precision positioning screen printing of permanent magnets, electronic panels, glass panels, ceramic substrates, metal signs, PCB boards and other products .

- Main parameters

| model | TX - 3050SF | Printing area | 300* 500mm |
| --- | --- | --- | --- |
| Platform size | 400 * 600mm | Maximum screen frame | 500* 900mm |
| Printing thickness | ≤ 200 mm | Printing accuracy | ± 0.02mm |
| Maximum number of idling prints | 1400 (PCS/H) | Printing pressure | 0.5-0.7Mpa |
| Power supply/power | 220V / 2.0KW | Gross Weight | 230 KG |
| Machine size | 900*600*1650mm | Packing size | 1100*750*1800mm |

★Main performance

①Mitsubishi PLC program control circuit, human-machine interface control, user-friendly operation.

②Vertical structure, the screen moves up and down vertically, and the printing head moves left and right.

③ The printing and screen lifting adopt independent driving sources, with coordinated movements and stable and adjustable speed.

④The Panasonic servo system drives the printing tool holder, the photoelectric eye controls the stroke and is adjusted independently.

⑤ Printing drive Panasonic servo motor, smooth operation.

⑥ The printing platform is equipped with precision fine-tuning handles in the horizontal and vertical directions, which enables accurate fine-tuning and convenient color registration.

⑦ There are three printing modes: manual/semi-automatic/fully automatic. The automatic printing time is digitally controlled (1~9 seconds).

⑧ Independently designed machine head lifting device, ink scraper/ink return knife, screen loading and unloading and screen cleaning are convenient.

⑨ The left and right screen clamp arms are equipped with plate spacing adjustment devices for easy adjustment.

⑩The angle and balance of scraping and ink return can be adjusted.

⑪The printing table adopts a vacuum suction work surface to facilitate the fixing of printing sheets .

★ Details display

★ Touch screen operation interface

★Configuration list

| Structure | project | Configuration Instructions |  |
| --- | --- | --- | --- |
| Basic structure | structure | Vertical | / |
|  | frame | Steel structure welding | self made |
|  | beam | A3 Steel Plate | self made |
|  | Sliding Bushing | Aluminum Alloy | self made |
|  | Linear square guide shaft | Linear square guide shaft | Taiwan Hiwin Guide Shaft |
|  | Base | Cast iron base | self made |
| Rack lifting | Lifting drive | Motor | Taiwan Beiji reducer motor |
|  | Lifting guide | Linear square guide shaft | Panasonic Servo Motor |
| Printing Drag | Printing driver | Servo System | Panasonic Servo System |
|  | Printing Guide | Linear square guide shaft | Taiwan Hiwin Guide Shaft |
| Shelf system | Screen clamping | Mechanical combination | Aluminum handle |
|  | Off-grid systems | none |  |
|  | Ink scraping system | pneumatic | SMC Cylinder |
| Platform system | Inhale | have | Vacuum adsorption work surface |
|  | Platform fine-tuning | Suction work surface | Scale display precision fine adjustment |
| Control System | Control Circuit | Taiwan Fengwei PLC | Can be printed multiple times. |
|  | Electrical components | Foreign famous brands | Omron relay, Schneider contactor, HLW induction switch |
|  | Pneumatic components | Solenoid valve | E-day solenoid valve and oil-water separator |
| Safety devices | Safety brake | Proximity switches | Red oxide brake pads |

★ Delivery list

| Serial number | Tool Name | Specification | unit | quantity | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | Power cord | Standard | root | 1 |  |
| 2 | Foot switch | Standard | indivual | 1 |  |
| 3 | Phillips screwdriver | Standard | Bundle | 1 |  |
| 4 | Adjustable wrench | Standard | Bundle | 1 |  |
| 5 | Hexagon socket wrench | Standard | set | 1 |  |
| 6 | Toolbox | Standard | indivual | 1 |  |
| 7 | User Manual |  | Book | 1 |  |

- After-sales service instructions

- support for all factory products .

- After installation and commissioning, free warranty for one year (excluding wearing parts and man-made damage) and lifelong maintenance.

- Conduct maintenance visits, on-site operations and technical training at irregular intervals.

- After-sales service content:

  - If the equipment fails, we will first communicate by phone. If the problem cannot be solved, our company will arrive at the site as soon as possible to solve it.

  - After-sales service includes not only equipment repair and maintenance, but also dedicated support for process technology. Our technicians can conduct on-site technical training and provide a full set of screen printing process technology, information and equipment solutions.

  - Professional technicians visit from time to time to track the usage of user equipment.

  - Repair and maintenance of screen printing equipment and auxiliary equipment.

  - Design and customize special product screen printing equipment and auxiliary equipment.

Corporate image

- Company Honors